Frequently Asked Questions About Lead

| Q: Why is lead a problem? | A: Lead is known to be harmful to human health if inhaled or ingested. The degree of harm depends upon the level of exposure from all sources. |
| --- | --- |
| Q: Where does lead come from? | Important sources of lead exposure include: ambient air, soil and dust (both inside and outside the home), food (which can be contaminated by lead in the air or in food containers), and water (from the corrosion of plumbing). On average, it is estimated that lead in drinking water contributes between 10 and 20 percent of total lead exposure in young children. Infants who consume mostly mixed formula can receive 40 to 60 percent of their exposure to lead from drinking water. Federal controls on lead in gasoline have significantly reduced people's exposure to lead. |
| Q: How can lead get into my drinking water? | A: Lead gets into your water after the water enters your water service lines or plumbing in your home. The source of lead in your home's water can be the service pipe, which can be made of lead, or solder or fixtures in your home's own plumbing. While UTILITY water is non-corrosive, some lead can be dissolved into your water if it is left stagnant in the pipes for more than six hours. |
| Q: Does my service line contain lead? | A: We have fairly complete records. Approximately NUMBER service lines have at least one lead component, our records show. There are some unknown lines that we will assume are lead until we can establish this with certainty. |
| Q: Does my home's age make a difference? | A: Yes. Up until the early 1930s, lead pipes were sometimes used for interior plumbing. Until the early 1950s, the UTILITY often used lead piping to connect water mains to residences. Solder used to join copper plumbing before 1988 may contain lead. New brass faucets and fittings may contain up to 8% lead, and can also leach lead even though they are technically considered "lead-free." We can tell you if the service connector used in your home or area is made of lead. Call us at (AREA) PHONE #. Lead levels decrease as a building ages. This is because as time passes, mineral deposits form a coating on the inside of the pipes This coating insulates the water from the solder. |

| Q: How can I have my water tested for lead? | A: Testing can be arranged through your local health department and costs around $30. Contact names and numbers are listed at the bottom of this document. Should you decide to utilize an independent laboratory not listed here, please be certain that the laboratory is certified by the Michigan Department of Environmental Quality (MDEQ) to test for lead in drinking water. |
| --- | --- |

| Q: What are the testing procedures? | A: Contact your local health department. In most cases, the department will provide sample containers along with instructions as to how you should draw your tap-water samples. If you collect the samples yourself, make sure you follow the health department's instructions exactly. Otherwise, the results might not be reliable. Make sure that the laboratory is following water sampling and analysis procedures certified by MDEQ. Be certain to take a "first draw" sample. |
| --- | --- |
| Q: What about lead in sources other than drinking water? | Important sources of lead exposure include: ambient air, soil and dust (both inside and outside the home), food (which can be contaminated by lead in the air or in food containers), and water (from the corrosion of plumbing). On average, it is estimated that lead in drinking water contributes between 10 and 20 percent of total lead exposure in young children. Federal controls on lead in gasoline have significantly reduced people's exposure to lead. |
| Q: What is the UTILITY doing about the problem of lead in household water? | A: We are taking three major actions to reduce your exposure to lead: - The UTILITY has undertaken corrosion control measures to minimize the amount of lead picked up by water contacting lead-bearing piping and plumbing. We continue to test treatment alternatives to further reduce the potential for lead corrosion in service lines and customer plumbing. - We have initiated a lead service line replacement program that will have removed all NUMBER lead service components in the areas we serve TIME FRAME years from now. - We are conducting a public education/awareness campaign targeted toward all of our customers by suggesting simple, effective methods to avoid lead exposure. |
| What measures can a customer take to reduce exposure? | A: The first step is to refrain from consuming water that has been in contact with your home's plumbing for more than six hours, such as overnight or during your work day. Before using water for drinking or cooking, "flush" the cold water faucet by allowing the water to run until you can feel that the water has become as cold as it will get. For instance, you might flush the toilet first thing in the morning or take a shower, then run your kitchen tap for 30 or more seconds before you draw any water for consumption. Also, you might draw water for plants or other non-consumptive use before flushing the tap. Following the intitial flush, whether toilet or shower, you must flush the tap 30 seconds for each drinking water faucet. This will void the service lines as well as the home plumbing. Once you have flushed a tap, you might fill one or more bottles with water and put them in the refrigerator for later use that day. The second step is to never cook with or consume water from the hot-water tap. Hot water dissolves more lead more quickly than cold water. So, do not use water taken from the hot tap for cooking or drinking, and especially not for making baby formula. (If you need hot water, draw water from the cold tap and heat it on the stove.) Use only thoroughly flushed water from the cold tap for any consumption. |
| Q: How much lead is too much? | A: Federal standards initially limited the amount of lead in water to 50 parts per billion (ppb). In light of new health and exposure data, EPA has set an action level of 15 ppb. If tests show that the level of lead in your household water is in the area of 15 ppb or higher, it is advisable - especially if there are young children in the home - to reduce the lead level in your tap water as much as possible. (EPA estimates that more than 40 million U.S. residents use water that can contain lead in excess of 15 ppb.) Note: One ppb is equal to 1.0 microgram per liter (µg/1) or 0.001 milligram per liter (mg/1). |

For More Information:

| County Health Department | (AREA) PHONE # |
| --- | --- |
| U.S. EPA Safe Drinking Water Hotline | (800) 426-4791 |
| U.S. EPA Lead Web Page | http://www.epa.gov/safewater/lead/leadfacts.html |

Definitions

Corrosion: A dissolving and wearing away of metal caused by a chemical reaction (in this case, between water and metal pipes, or between two different metals). First Draw: The water that immediately comes out when a tap is first opened. Flush: To open a cold-water tap to clear out all the water which may have been sitting for a long time in the pipes. In new homes, to flush a system means to send large volumes of water gushing through the unused pipes to remove loose particles of solder and flux. (Sometimes this is not done correctly or at all). Flux: A substance applied during soldering to facilitate the flow of solder. Flux often contains lead and can, itself, be a source of contamination. Naturally soft water: Any water with low mineral content, lacking the hardness minerals calcium and magnesium. Public Water System: Any system that supplies water to 25 or more people or has 15 or more service connections (buildings or customers). Service Connector: The pipe that carries tap water from the public water main to a building. In the past these were often made of lead. Soft water: Any water that is not "hard." Water is considered to be hard when it contains a large amount of dissolved minerals, such as salts containing calcium or magnesium. You may be familiar with hard water that interferes with the lathering action of soap. Solder: A metallic compound used to seal joints in plumbing. Until recently, most solder contained about 50 percent lead.